| Information Delivery Specifications IDS |  |
| --- | --- |
| IDS Bingo Berlin This IDS contains specifications for every facet. Every facet has specifications that are simple to verify basic checkers as well as specifications containing regular expressions. The results can be checked on the Bingo scorecard. |  |
| Version: | 1 |
| Date: | 24/09/2025 |
| Author Email : | menno@aronsengelauff.nl |
| Milestone: | Technical Review |
| Purpose: | IDS written for 'ILSOenE_PoC-model.ifc' to test IDS tools for creating and validating specifications. |
| Copyright: Arons en Gelauff architecten / BIMvoice |  |

Created with usBIM.IDSeditor by ACCA software S.p.A.

# Specifications

## ATTRIBUTE: IfcProjectedCRS must be a valid EPSG code - ATT01

Checks if the model is placed onto the correct coordinate system https://standards.buildingsmart.org/IFC/RELEASE/IFC4_3/HTML/lexical/IfcProjectedCRS.htm

IFC versions: IFC4X3_ADD2 IFC4

The model MUST contain entities that have

-	IFC class IFCPROJECTEDCRS

that MEET the following requirements

-	MUST HAVE attribute Name = EPSG:28992 (Instructions: Choose the EPSG-code for your model here: https://epsg.io/28992Change it to something else to see if the specification works.)

Instructions

If your tool does not support this entity, skip this specification.

Result

The specification should pass. Try to change the EPSG to something else to find out if you get a mistake.

## ATTRIBUTE: Attributes in IfcMapConversion must be present - ATT02

Checks if the model is placed on coordinates https://standards.buildingsmart.org/IFC/RELEASE/IFC4_3/HTML/lexical/IfcMapConversion.htm

IFC versions: IFC4X3_ADD2 IFC4

The model MUST contain entities that have

-	IFC class IFCMAPCONVERSION

that MEET the following requirements

-	MUST HAVE attribute Eastings (Instructions: Attribute must exist)

-	MUST HAVE attribute Northings = 422691.5309 (Instructions: Attribute must exist)

-	MUST HAVE attribute OrthogonalHeight (Instructions: Attribute must exist)

Instructions

If your tool does not support this entity, skip this specification.

Result

The specification should pass. When it fails the Northings you can check the notation. Did you enter meters, a point or a comma. Your tool either does not support this or do you need to change something in the notation.

## ATTRIBUTE: Attributes in IfcMapConversion cannot be 0 - ATT03

Checks if the model is placed on coordinates https://standards.buildingsmart.org/IFC/RELEASE/IFC4_3/HTML/lexical/IfcMapConversion.htm

IFC versions: IFC4X3_ADD2 IFC4

The model MUST contain entities that have

-	IFC class IFCMAPCONVERSION

that MEET the following requirements

-	MUST HAVE attribute Eastings < 0 (Instructions: Attribute must be greater than 0(the coordinates must be set but cannot be at 0,0,0 for the model to be at the correct location)(This test is less than 0 in order to generate a mistake))

-	MUST HAVE attribute Northings > 0 (Instructions: Attribute must be greater than 0(the coordinates must be set but cannot be at 0,0,0 for the model to be at the correct location))

-	MUST HAVE attribute OrthogonalHeight ≥ 1.25 (Instructions: Attribute must be greater than or equal to 1,25m(Actually it should just be 1.25m but this is to test the different bounds.))

Instructions

When bounds do not work, skip this specification.

result

the specification should pass.

## ATTRIBUTE: list of correct naming of building storeys - ATT04

validates the naming of building storeys in a list https://standards.buildingsmart.org/IFC/RELEASE/IFC4_3/HTML/lexical/IfcBuildingStorey.htm

IFC versions: IFC4X3_ADD2 IFC4 IFC2X3

The model MUST contain entities that have

-	IFC class IFCBUILDINGSTOREY

that MEET the following requirements

-	MUST HAVE attribute Name with value = 00 ground floor (or:  = 01 first floor;  = 02 second floor)

Instructions

Add a list of names in the Requirements

Results

The model contains 3 building storeys: 00 nulth floor, 01 first floor and 02 second floor. 00 nulth floor is obviously wrong. The specification should give you this IfcBuildingStorey as a result.

## ATTRIBUTE: simple RegEx list of correct naming of building storeys - ATT05

validates the naming of building storeys in with a RegEx pattern https://standards.buildingsmart.org/IFC/RELEASE/IFC4_3/HTML/lexical/IfcBuildingStorey.htm

IFC versions: IFC4X3_ADD2 IFC4 IFC2X3

The model MUST contain entities that have

-	IFC class IFCBUILDINGSTOREY

that MEET the following requirements

-	MUST HAVE attribute Name matching the pattern 00 ground floor|01 first floor|02 second floor

Instructions

Add a list of names in the Requirements by use of regular expression

Results

The model contains 3 building storeys: 00 nulth floor, 01 first floor and 02 second floor. 00 nulth floor is obviously wrong. The specification should give you this IfcBuildingStorey as a result.

## ATTRIBUTE: complex RegEx of correct naming of building storeys - ATT06

validates the naming of building storeys in with a RegEx pattern https://standards.buildingsmart.org/IFC/RELEASE/IFC4_3/HTML/lexical/IfcBuildingStorey.htm

IFC versions: IFC4X3_ADD2 IFC4 IFC2X3

The model MUST contain entities that have

-	IFC class IFCBUILDINGSTOREY

that MEET the following requirements

-	MUST HAVE attribute Name matching the pattern [0-2][0-2].*floor

Instructions

Add a list of names in the Requirements by use of regular expression

Results

The model contains 3 building storeys: 00 nulth floor, 01 first floor and 02 second floor. 00 nulth floor is obviously wrong. However, with this regular expression you will not find it.

## ENTITY: Walls must have allowed Predefined Types - ENT01

These are the Predefined Types. Be aware of not allowed types. https://standards.buildingsmart.org/IFC/RELEASE/IFC4_3/HTML/lexical/IfcWallTypeEnum.htm

IFC versions: IFC4X3_ADD2 IFC4

The model MAY contain entities that have

-	IFC class IFCWALL

that MEET the following requirements

-	MUST HAVE IFC class IFCWALL with predefined type ELEMENTEDWALL (or: MOVABLE; PARAPET; PARTITIONING; PLUMBINGWALL; RETAININGWALL; SHEAR; SOLIDWALL; WAVEWALL; USERDEFINED; NOTDEFINED) (Instructions: Do not allow for POLYGONAL and STANDARD as they are deprecated in the current Schema.)

Instructions

Find Walls and check for all of the Predefined Types that are allowed in the Schema.

Use the required cardinality: minOccurs="1" maxOccurs="unbounded".

Results

The red walls are an IfcWall with predefined type: STANDARD. This enumeration is deprecated in IFC4x3 so this is not in the list and should be the result of this specification.

## ENTITY: Walls may have allowed Predefined Types - ENT02

These are the Predefined Types. Be aware of not allowed types. https://standards.buildingsmart.org/IFC/RELEASE/IFC4_3/HTML/lexical/IfcWallTypeEnum.htm

IFC versions: IFC4X3_ADD2 IFC4

The model MUST contain entities that have

-	IFC class IFCWALL

that MEET the following requirements

-	MUST HAVE IFC class IFCWALL with predefined type ELEMENTEDWALL (or: MOVABLE; PARAPET; PARTITIONING; PLUMBINGWALL; RETAININGWALL; SHEAR; SOLIDWALL; WAVEWALL; USERDEFINED; NOTDEFINED) (Instructions: Do not allow for POLYGONAL and STANDARD as they are deprecated in the current Schema.)

Instructions

Find Walls and check for all of the Predefined Types that are allowed in the Schema.

Use the optional cardinality: minOccurs="0,"maxOccurs="unbounded".

Results

The red walls are an IfcWall with predefined type: STANDARD. This enumeration is deprecated in IFC4x3 so this is not in the list and should be the result of this specification.

## ENTITY: Walls must not have Predefined Types that are not allowed - ENT03

- Walls may not have a Predefined Type that is deprecated. https://standards.buildingsmart.org/IFC/RELEASE/IFC4_3/HTML/lexical/IfcWallTypeEnum.htm

IFC versions: IFC4X3_ADD2 IFC4

The model MUST NOT contain entities that have

-	IFC class IFCWALL with predefined type POLYGONAL (or: STANDARD)

that MEET the following requirements

- 	...

Instructions

Find Walls and check for all of the Predefined Types that are not allowed in the Schema.Use the prohibited cardinality: minOccurs="0" maxOccurs="0".IDS documentation reads: requirements are ignored in the verification of models for the prohibited cardinality.

Results

The red walls are an IfcWall with predefined type: STANDARD. This enumeration is deprecated in IFC4x3 so this is not in the list and should be the result of this specification.

## CLASSIFICATION: walls must have CCI classification - CL01

Checks if the classification is present.This specification has an Entity as applicability.

IFC versions: IFC4X3_ADD2 IFC4 IFC2X3

The model MUST contain entities that have

-	IFC class IFCWALL

that MEET the following requirements

-	MUST HAVE classification CCI Construction

-   URI: https://identifier.buildingsmart.org/uri/molio/cciconstruction/1.0/class/L-AD

Instructions

Find all walls, they need to have a CCI classification

Results

The red walls do not have a CCI Classification code.

## CLASSIFICATION: concrete walls must have CCI classification - CL02

Checks if the classification is correct. This specification has an attribue as applicability as well as an Entity.

IFC versions: IFC4X3_ADD2 IFC4 IFC2X3

The model MUST contain entities that have

-	IFC class IFCWALL

-	attribute Name matching the pattern .*beton.*

that MEET the following requirements

-	MUST HAVE classification CCI Construction.L-AD

-   URI: https://identifier.buildingsmart.org/uri/molio/cciconstruction/1.0/class/L-AD

Instructions

Find all elements containing the word 'beton' (Dutch for concrete) in the Attribute 'Name', they need to have a CCI classification LA-D.

Result

The red walls have a name, 22_cellenbeton_100,  that contains the word ‘beton’. The CCI classification is L-B instead of L-AD.

## CLASSIFICATION: CCI classification L-Ad must be a wall. - CL03

Reversed specification that wlil find a Slab that has a Wall-code.

IFC versions: IFC4X3_ADD2 IFC4 IFC2X3

The model MUST contain entities that have

-	classification CCI Construction.L-AD

that MEET the following requirements

-	MUST HAVE IFC class IFCWALL

Instructions

Find all elements with classification L-AD, they must be walls.

Result

The red slab has a CCI classification of L-AD but that’s wrong. L-AD is walls.

## PROPERTY: IsExternal in Requirement - PROP01

https://standards.buildingsmart.org/IFC/RELEASE/IFC4_3/HTML/property/IsExternal.htm

- If IsExternal in the Pset, this entity should have it defined.

- IFC versions: IFC4X3_ADD2 IFC4 IFC2X3

The model MUST contain entities that have

-	IFC class IFCWALL (or: IFCSLAB; IFCRAILING; IFCCOVERING; IFCROOF; IFCSPATIALZONE; IFCSTAIR; IFCPLATE; IFCBEAM; IFCCHIMNEY; IFCRAMP; IFCJUNCTIONBOX; IFCCOLUMN; IFCMEMBER; IFCSPACE; IFCBUILDINGELEMENTPROXY; IFCDOOR; IFCWINDOW; IFCSHADINGDEVICE; IFCSIGN; IFCCURTAINWALL)

that MEET the following requirements

-	MUST HAVE property IsExternal of PSet matching the pattern Pset_.*Common (IFCBOOLEAN)

Instructions

Find out if all entities that have IsExternal in their Common Psets actually have this Property defined.

Result

One space does not have IsExternal set in the Pset_SpaceCommon.

## PROPERTY: IsExternal in Applicability - PROP02

https://standards.buildingsmart.org/IFC/RELEASE/IFC4_3/HTML/property/IsExternal.htm

IFC versions: IFC4X3_ADD2 IFC4 IFC2X3

The model MUST contain entities that have

-	property IsExternal of PSet Pset_WallCommon (IFCBOOLEAN)

that MEET the following requirements

-	MUST HAVE IFC class IFCWALL (Instructions: For this specification, IfcWallType has been left out to generate some failures.)

Instructions

IsExternal in Pset_WallCommon should be IfcWall

Result

This should always be correct right? If your tool supports the validation of Types, you will get mistakes. If not, the specification will pass.

## PROPERTY: External Railing should have IsExternal = True - PROP03

https://data.ketenstandaard.nl/publications/nlsfb/2021/781eb5a2-6424-483d-b050-73a7bbff5775

IFC versions: IFC4X3_ADD2 IFC4 IFC2X3

The model MAY contain entities that have

-	IFC class IFCRAILING

-	classification NL-SfB Tabel 1.34.12

that MEET the following requirements

-	MUST HAVE property IsExternal of PSet Pset_RailingCommon (IFCBOOLEAN) = true (Instructions: DataType: IFCBOOLEAN, URI: https://identifier.buildingsmart.org/uri/buildingsmart/ifc/4.3/prop/IsExternal

Instructions

Find the Railing with NL-SfB ClassificationCode: 34.12: balustrades, handrails; balustrades, outer balustrades'. Outer balustrades should have the property IsExternal = True.

Result

IsExternal in Pset_RailingCommon in the Railing is set to False, it should be True.

## PROPERTY: ticket vending machine is eating my money! - PROP04

https://standards.buildingsmart.org/IFC/RELEASE/IFC4_3/HTML/lexical/IfcElectricAppliance.htm

IFC versions: IFC4X3_ADD2

The model MAY contain entities that have

-	IFC class IFCELECTRICAPPLIANCE.VENDINGMACHINE

that MEET the following requirements

-	MUST HAVE property TicketVendingMachineType of PSet Pset_TicketVendingMachine (IFCLABEL) = TICKETVENDINGMACHINE (Instructions: DataType: IFCLABELTicketVendingMachineType is a enumerated property, chose the correct one from this list: https://standards.buildingsmart.org/IFC/RELEASE/IFC4_3/HTML/lexical/PEnum_TicketVendingMachineType.htm

URI: https://identifier.buildingsmart.org/uri/buildingsmart/ifc/4.3/prop/TicketVendingMachineType

-	MUST HAVE property TicketStuckRatio of PSet Pset_TicketVendingMachine (IFCRATIOMEASURE) < 10 (Instructions: DataType: IFCRATIOMEASURE https://standards.buildingsmart.org/IFC/RELEASE/IFC4_3/HTML/lexical/Pset_TicketVendingMachine.htm

URI: https://identifier.buildingsmart.org/uri/buildingsmart/ifc/4.3/prop/TicketStuckRatio

-	MUST HAVE property MoneyStuckRatio of PSet Pset_TicketVendingMachine (IFCRATIOMEASURE) ≥ 50 (Instructions: DataType: IFCRATIOMEASURE https://standards.buildingsmart.org/IFC/RELEASE/IFC4_3/HTML/lexical/Pset_TicketVendingMachine.htm

URI: https://identifier.buildingsmart.org/uri/buildingsmart/ifc/4.3/prop/MoneyStuckRatio

-	MUST HAVE property Manufacturer of PSet Pset_ManufacturerTypeInformation (IFCLABEL) = WPS (Instructions: DataType: IFCLABEL https://standards.buildingsmart.org/IFC/RELEASE/IFC4_3/HTML/lexical/Pset_ManufacturerTypeInformation.htm

URI: https://identifier.buildingsmart.org/uri/buildingsmart/ifc/4.3/prop/Manufacturer

Instructions

Find the Ticket vending machine and add Properties Manufacturer, TicketVendingMachineType, TicketStuckRatio and MoneyStuckRatio. Set the ratios as bounds.

Result

TicketStuckRatio should generate a mistake. Test a few values that are equal to, lesser or greater than 50 to see how your specification works.

## MATERIAL: specific element must have a specific name - MAT01

Validating materials is usually never done like this. But if a tool does not support RegEx, this is a simple test.

IFC versions: IFC4X3_ADD2 IFC4

The model MUST contain entities that have

-	IFC class IFCWALL.SOLIDWALL

-	attribute Name = 21_metselwerk type 1_100

that MEET the following requirements

-	MUST HAVE material steenachtig_baksteen_steen_roodroze

URI: https://identifier.buildingsmart.org/uri/nkt/naakt/2.4/class/steenachtig_baksteen_steen

Instructions

Find walls with name '21_metselwerk type 1_100', the Material should be steenachtig_baksteen_steen_roodroze

Result

The correct wall should be found

## MATERIAL: brickwork should start with steenachtig_baksteen_steen - MAT02

Usually you need to filter elements and then, the materialname must have a specific syntax ore a specific word in the name. Regular expressions are key in these type of specifications.

IFC versions: IFC4X3_ADD2 IFC4

The model MUST contain entities that have

-	IFC class IFCWALL.SOLIDWALL

-	attribute Name matching the pattern .*metselwerk.*

that MEET the following requirements

-	MUST HAVE material matching the pattern steenachtig_baksteen_steen.*

Instructions

Find walls with a name that contains 'metselwerk', the Material should start with steenachtig_baksteen_steen

Result

The correct walls will be found.

## MATERIAL: a materialname must have a specific property - MAT03

https://standards.buildingsmart.org/IFC/RELEASE/IFC4_3/HTML/lexical/PEnum_AssemblyPlace.htm

IFC versions: IFC4X3_ADD2 IFC4

The model MUST contain entities that have

-	material matching the pattern (beton|gips|hout|ntb|generiek)_.*_prefab-element.*

that MEET the following requirements

-	MUST HAVE property AssemblyPlace of PSet Pset_ManufacturerTypeInformation (IFCLABEL) = FACTORY

URI: https://identifier.buildingsmart.org/uri/buildingsmart/ifc/4.3/prop/AssemblyPlace

Instructions

Find elements with a materialname containing 'prefab-element'. They should have the AssemblyPlace property set as FACTORY in Pset_ManufacturerTypeInformation

Result

The concrete slab is a prefab element, assembled in the Factory. The gypsum boards are prefabicated pieces but do not have the property AssemblyPlace set.

## PARTOF: MEP part of an IfcDistributionsystem - PO01

IFC versions: IFC4X3_ADD2 IFC4

The model MUST contain entities that have

-	IFC class IFCPIPEFITTING (or: IFCPIPESEGMENT)

that MEET the following requirements

-	MUST HAVE relationship IFCRELASSIGNSTOGROUP with IFC class IFCDISTRIBUTIONSYSTEM.DOMESTICCOLDWATER

Instructions

Find some pipes, they must be part of an IfcDistributionSystem

Result:

Some pipesegments and fittings are not part of the IfcDistributionSystem

## PARTOF: A smokesensor must be in a space - PO02

IFC versions: IFC4X3_ADD2 IFC4

The model MUST contain entities that have

-	IFC class IFCSENSOR.SMOKESENSOR

that MEET the following requirements

-	MUST HAVE relationship IFCRELCONTAINEDINSPATIALSTRUCTURE with IFC class IFCSPACE

Instructions

Find the IfcSensor.SMOKESENSOR that must be contained it a spatial structure, an IfcSpace

Result

The Smokedetector is part of a space.

(In IDS version 1.0 we cannot further define what space it should be part of.)

## PARTOF: A building must be build on a site - PO03

IFC versions: IFC4X3_ADD2 IFC4 IFC2X3

The model MUST contain entities that have

-	IFC class IFCBUILDING

that MEET the following requirements

-	MUST HAVE relationship IFCRELAGGREGATES with IFC class IFCSITE

Instructions

Find the IfcBuilding, it should aggregate into an IfcSite

Result

The building has a relation to the site it is on.

## ENTITY: Walls must not have Predefined Types that are not allowed (illegal specification)) - INVALID01

These are the Predefined Types. Be aware of not allowed types. https://standards.buildingsmart.org/IFC/RELEASE/IFC4_3/HTML/lexical/IfcWallTypeEnum.htm

IFC versions: IFC4X3_ADD2

The model MUST NOT contain entities that have

-	IFC class IFCWALL

that MEET the following requirements

-	MUST HAVE IFC class IFCWALL with predefined type POLYGONAL (or: STANDARD) (Instructions: Do not allow for POLYGONAL and STANDARD as they are deprecated in the current Schema.)

Instructions

Find Walls and check for all of the Predefined Types that are not allowed in the Schema.Use the prohibited cardinality: minOccurs="0" maxOccurs="0".IDS documentation reads: requirements are ignored in the verification of models for the prohibited cardinality. This is not really well implemented in most tools yet? Tools usually understand this method and will read the requierements anyway.

Result

Red walls are IfcWall.STANDARD.

## PROPERTY: ticket vending machine is eating my money! - INVALID02

https://standards.buildingsmart.org/IFC/RELEASE/IFC4_3/HTML/lexical/IfcElectricAppliance.htm

IFC versions: IFC4X3_ADD2

The model MAY contain entities that have

-	IFC class IFCELECTRICAPPLIANCE.VENDINGMACHINE

that MEET the following requirements

-	MUST NOT HAVE property MoneyStuckRatio of PSet Pset_TicketVendingMachine (IFCRATIOMEASURE) > 10 (Instructions: DataType: IFCRATIOMEASUREhttps://standards.buildingsmart.org/IFC/RELEASE/IFC4_3/HTML/lexical/Pset_TicketVendingMachine.htm, URI: https://identifier.buildingsmart.org/uri/buildingsmart/ifc/4.3/prop/MoneyStuckRatio)

-	MUST NOT HAVE property TicketStuckRatio of PSet Pset_TicketVendingMachine (IFCRATIOMEASURE) < 50 (URI: https://identifier.buildingsmart.org/uri/buildingsmart/ifc/4.3/prop/TicketStuckRatio)

Instructions

Find the Ticket vending machine and add Properties Manufacturer, TicketVendingMachineType, TicketStuckRatio and MoneyStuckRatio.Set the ratios as bounds.

Result

An alternative way of validating a model on restrictions. Something should be less than 10… or somehting cannot be greater or equal to 10. Same thing, use what works best for you and the tools you are using.

## PARTOF: Door fills wall - INVALID03

IFC versions: IFC4X3_ADD2

The model MUST contain entities that have

-	IFC class IFCDOOR

that MEET the following requirements

-	MUST HAVE relationship IFCRELVOIDSELEMENT IFCRELFILLSELEMENT with IFC class IFCWALL

Result

Some elements are nested elements that do not void or fill an element. The one door in the interior wall does. IFCRELVOIDSELEMENT IFCRELFILLSELEMENT is a list and the IDS audit tool generates a mistake since you should choose one of the two. However, the tools used to create anvalidate this do not agree with this somehow.